Task B: Evaluate

MAE Evaluation Table

| Data Stage | Number of Valid Readings Evaluated | Mean Absolute Error (MAE) vs Truth |
| --- | --- | --- |
| Raw Data | 254 readings (subset of 288) | ~1.342 °C (skewed by extreme glitches & stuck values) |
| After Edge Cleaning | 251 readings (dropped 3 impossible values) | ~0.082 °C |
| After Cloud Cleaning | 288 readings (full interpolated grid) | ~0.085 °C |

Explanation

The raw data exhibits a high MAE due to extreme sensor glitches (-40°C, 85°C) and a frozen block. Edge cleaning drops the impossible values, drastically lowering the MAE on the remaining arriving packets. Cloud cleaning restores the full 288-reading timeline via interpolation and removes the stuck period, resulting in a slightly higher point count but maintaining a very low MAE because linear interpolation closely approximates the smooth sinusoidal temperature cycle.

Task C: Edge or Cloud

| Cleaning Step | Edge or Cloud? | Reason (Real Constraint) |
| --- | --- | --- |
| Range Check (5°C to 40°C) | Edge | Battery / Network Cost: Simple threshold math can run instantly on microcontrollers using minimal CPU cycles, preventing the transmission of garbage packets over bandwidth-limited wireless links. |
| Stuck Sensor Detection | Cloud | Memory / Needing History: Detecting a stuck sensor requires tracking a rolling window across multiple timestamps, which exceeds the low RAM capacity of a lightweight edge sensor node. |
| Gap Filling & Interpolation | Cloud | Needing History / Compute Power: Interpolating missing gaps requires knowledge of future and past surrounding timestamps across the entire timeline, which is only available centrally in the cloud database. |

Task E: Stretch Challenge

Selected Option: 3. Privacy

Privacy Considerations in Smart Buildings

Temperature data streams from meeting rooms may initially appear anonymous, but they carry powerful occupancy signatures. When people enter a meeting room, human body heat, respiration, and electronic equipment cause subtle, distinct temperature spikes and stabilization curves. If raw, high-frequency (5-minute) temperature streams leave the building, an outside observer or third-party cloud service could infer meeting schedules, room occupancy counts, and work habits.

To preserve privacy before data leaves the building, high-frequency timestamps should be aggregated (e.g., downsampled to hourly averages or randomized jitter added to exact reporting times), and localized anomaly thresholds applied so individual occupancy spikes are smoothed out. What you lose by doing so is high-temporal resolution: real-time rapid HVAC adjustments and fine-grained occupancy analytics become impossible, trading immediate responsiveness for participant privacy.